Communication Protocols

We begin by looking at the protocols that are used for modem communications, typically between a home or small business user and an Internet Service Provider’s server.  Sending and receiving computers must use the same modem protocols and settings if they are to communicate reliably.

Protocols are sets of rules about how the two devices are to communicate.  So, for example, if a modem is set to use 8 data bits, no parity bit and 1 stop bit, the other modem will not be able to interpret the data unless it uses the same settings.  In addition to this, both modems must operate at the same speed.  This is why, for example, when connecting to the Internet, the connect speed may vary, up to the maximum of the users modem.  The connect speed depends on the modem speed at the ISP’s site, when the dial-up connection is made.

For communication between two devices, the protocol must establish as a minimum:

- The data transmission rate (measured in bits per second).  If the devices have different maximum transmission rates, the rate is dictated by the slower of the two.

- Whether both devices can transmit to each other in full-duplex mode or only half-duplex mode.

- Whether synchronous (data is sent in a stream with the sender and receiver synchronised to identify when one character ends and another starts) or asynchronous (characters are sent at irregularly timed intervals, with start and stop bits to separate characters) transmission is to be used.

A protocol may also specify the form of handshaking, the method of data flow control and mechanisms for error control and data compression.

Handshaking, flow control

Handshaking means that communicating devices must have a common method of determining their presence and readiness to communicate.  Although the details vary, a handshake can be illustrated simply by the following example:

- Device A wants to communicate with device B

- Device A signals “Hello device B, are you there?” to which device B replies

- “Yes, I am here device A”.  Finally, device A responds

- “ I see that you are there device B”.

The handshake should also define when each device has finished sending or receiving a message.

A flow control mechanism is needed to ensure that data transmission flows smoothly, that the communications channel is not overloaded and that the transmitting device does not send data more quickly than the receiver can handle.

All modern modems include hardware to do this, but if both modems do not support hardware flow control, then software (called XON/XOFF) must be used.  When the receiving device’s buffer is full, it sends an XOFF signal to the sending device, which then pauses transmission.  Once the receiving device has cleared space in its buffer and is ready to receive again, it sends an XON signal to the sending device.

Error control and data compression

Although digital signals are more stable than their analogue equivalent, errors will occur and the communicating devices must have a means of detecting and re-transmitting them correctly.  For transmission over analogue links (which are the norm for local connections to the public telephone network) error detection and correction is even more important and all modem error protocols are built into the hardware.  Data compression reduces the volume of data, allowing its speedier transmission.

Protocols can be implemented by software or hardware.  Dedicated network devices such as modems use hardware implementation, but general-purpose computers that have not been designed for connection to a particular network may have to set up transmission protocols through the communications software.

Examples of protocols

TCP/IP  (Transmission Control Protocol/Internet Protocol) is the standard for communication between Internet host computers and is probably the most well known.  It is actually made up of a suite of several protocols.  The importance of the Internet for network communications means that all modern operating systems provide support for TCP/IP, including Macintosh computer systems and Novell’s Netware network operating system.

Unix is the standard operating system for most Internet host computers and has the TCP/IP protocol built in.

SMTP  stands for Simple Mail Transfer Protocol and is the standard for e-mail transmissions between servers and clients on the Internet.

UDP  The User Datagram Protocol is used for the broadcast of data over IP networks but unlike TCP, does not require that a connection be first established between sender and receiver.  In other words, the data is transmitted without any guarantee that it will reach its destination.  For this reason, UDP is referred to as a connectionless protocol.

TCP is a connection-orientated protocol, which means that the link between devices must be established before communication can begin.  Ethernet local area networks also operate on a connectionless, broadcast principle.

X.25  This is a CCITT protocol for packet switching networks.  X.25 packet switched networks allow remote devices to communicate with each other across high-speed digital links without the expense of individual leased lines.  Packet switching is a technique whereby the network routes individual packets of data between different destinations based on addressing within each packet.

FTP  File Transfer Protocol allows large amounts of information to be transferred between two computers very conveniently.  FTP originated with the Unix operating system and was used to transfer files of all types between remote computer systems.  A popular application of FTP is for the uploading and downloading of Web site pages.

ASCII  This is only appropriate for text files, which contain no control characters.  It cannot be used to transfer files produced with a word processor, spreadsheet or graphics packages.  Neither can it transfer command (COM) or executable (EXE) files, or files in compressed format (ZIP files).  Apart from this, the protocol is not good at controlling errors.

Xmodem  This is a file transfer protocol used for asynchronous communication and is commonly used in communications packages.  Its ability to find and correct errors makes it suitable for the transfer of files that must retain their integrity, such as program files.

Zmodem  This is one of the most advanced protocols, being much faster than Xmodem.  Its error correction controls are absolutely reliable.

CCITT V42bis  This protocol includes data compression (through encoding, the data is reduced in volume) technique and error detection and correction.  Both the sending and receiving modem must possess the error correction facility.