Parallel & Serial Transmission

Parallel transmission

Devices differ in the ways they communicate or talk with each other.  Parallel transmission is quicker than sending the bits one after another (as in serial transmission) but is only practicable over short distances.  Communication between a computer and its nearby peripherals can be carried out using parallel transmission, which is particularly important where high-speed devices, such as disk or tape units, are concerned.  Most stand alone computers use parallel transmission to communicate with a nearby printer.

Sender                                                                                                                  Receiver

1 0 1 1 0 0 1 1 0 0 0 1 0 1 1 1 0

0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 1

0 1 0 1 1 0 0 1 0 1 0 1 0 1 1 0 0

1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1 1

Parallel transmission

Serial transmission

Serial transmission is the only practicable method for communication over long distances.

0100110001010110111010010010101

Serial transmission

The principle problems with serial transmission concern the identification of the individual bits which make up a data stream and the division of bit groupings into words.  A word is a given number of bits used as a standard unit.  For example, a character within a data stream may comprise 7 or 8 bits, depending on the coding system in use.  A commonly used coding system is ASCII, which uses 7 bits per character, plus a parity bit for error checking.  There are two techniques for achieving this separation of bits and words:

- Asynchronous transmission

- Synchronous transmission

Asynchronous serial transmission

When a sending device transmits characters at irregular intervals, like a keyboard for example, it is said to be transmitting asynchronously.

Although the characters are not sent at regular intervals, the bits within each character must be sent at regular timed intervals.  An example of asynchronous character format is shown below:

Direction of flow

Receivers clock

starts

1 Mark level

0          1         0          1          1        0         0         1         0     1

0 Space level

7 data bits

Stop bit

Start bit                                                                             Parity bit

It can be seen that the line has two electrical states, representing 1 and 0.  To allow the receiver to identify individual characters, the transmitter signals the beginning of a character with a start bit and the end with a stop bit.  In this diagram the character comprises 7 bits.  It also shows the use of a parity bit which was a simple method of detecting errors caused by noise on the transmission line.

The sophisticated error detection and correction controls provided by today’s high speed modems make the use of the single parity bit unnecessary.

When no data is being transmitted the line is in its idle state, a condition maintained by a logical 1 output from the transmitter.  This is known as the mark level.  When it is ready to send a character, the transmitter outputs a logical 0 signal, which drops the signal to space.  Sensing this state, the receiver starts its clock after one bit time (for example, on a line operating at 9600 bits per second, one bit time is 1/9600ths of a second) and counts off and samples the regularly timed bits that form a character.  Even if the receivers clock is not exactly in time with that of the transmitter, the bit-time is sufficient to allow each bit to be sampled and identified.  If the receiver is out of synchronisation by no more than 5% then the system should still operate effectively.

When a stop bit is reached and the line returns to the mark level, the receiver switches back to its listening state.  Although the transmitter of individual bits requires fairly closely matched time control by transmitter and receiver, the presence of start and stop bits for each character permits the time interval between characters to be irregular, or asynchronous.

Modem frequency spectrum

A modem which uses frequency modulation will use different frequencies for logical 0 (space) and logical 1 (mark).  For example, an early modem standard, CCITT V21, used the following frequencies to provide full duplex, asynchronous transmission at 300 bps.

Modem operation                                                      Space (logical 0)          Mark (logical 1)

Originate transmission    Transmit frequency                   1180 Hz                       980 Hz

Originate transmission    Receive frequency                     1850 Hz                     1650 Hz

Answer                          Transmit frequency                    1850 Hz                     1650 Hz

Answer                          Receive frequency                      1180 Hz                      980 Hz

Parity bit

The ASCII code is a useful 7-bit code for plain text transmission, but its error control facilities are primitive.  An additional bit, known as the parity bit is attached to each character and can be used to detect single bit errors which may occur during data transfer.  There are two types of parity, odd and even, though it is of little significance which is used.

Asynchronous modem settings

The connection tab for modem settings in the Windows operating systems show each character comprising 8 data bits and 1 stop bit.  Because the modem uses its own error controls, a parity bit is not required (see below).

Baud rate, bandwidth and data compression

The term baud rate refers to the line speed operating between two modems.  For example, a 28,800 baud connection can transfer data at a rate of 28,800 bits per second (bps).  The term bandwidth refers to the amount of data that can be pushed through a medium at any given time and is measured in bits per second (bps).  For example, a modem operating at 56K bits has twice the bandwidth of one operating at 28 Kbits.  Bandwidth is important for modern communications because of the increased use of networks for the transmission of multimedia images.  By using data compression, a modem can transfer a given amount of data with fewer bits, thus improving the rate of data throughput.  A modem can receive data through the COM port (a PCs RS232 interface) at around four times the rate of the actual line.  So, for example, using a connect speed of 28800 baud, a modem could accept data from the COM port at 115,200 bps (4 x 28,800).

COM Port set at speed 115200 bps.

The fastest modems will operate at 56 K (56 x 1024) bits per second and with data compression can achieve much higher effective data transfer rates (throughput).  However, to achieve this maximum speed, the receiving modem must be able to operate at the same speed and use the same compression protocols (rules) to uncompress the data.

As part of the data compression process, a modem will also strip out the start and stop bits which are still attached to each character for transmission through the COM port to the modem.  This reduces the wasted transmission time which would otherwise be taken up by start and stop bits for each character.  In earlier systems, these transmission overheads significantly reduced data throughput.  For example, an early 2400 baud modem could transfer only 240 characters per second

because each character required a minimum of 10 bits (1 start bit, 1 stop bit and 8 data bits).  Without the start and stop bits, the rate would be 300 characters per second (2400/8 = 300)

Although the technological advances in modems have permitted a radical increase in asynchronous data transmission performance, the sending PC is still having to attach start and stop bits to each character and there are circumstances when synchronous transmission provides a more efficient alternative.

V.90 Modem standard

The most recent standard for analogue modems enables connection to the Internet at speeds of up to 56 Kbps (V.90 modems are labelled as such, but the theoretical maximum speed is 54 Kbps, because of technical restrictions).  Previous modem standards were developed on the assumption that analogue modems would be used at each end of a link, and that the analogue/digital conversion processes at either end would limit the connect speed to 33.6 Kbps.  In fact, most computers are communicating via the Internet, using Internet Service Providers (ISP) who have digital connections to the PSTN.  Thus, analogue/digital conversions are only being carried out at one end of a link, for the analogue connection between the Internet users computer and the local digital exchange to the PSTN.

Router & Internet

Computer                                      Digital                                Internet Service

Exchange                               Provider

Digital signal              Phone socket                             Digital

Signal

V90 Modem

Analogue signal

PSTN

Connection to the Internet using V.90 modem

When a V.90 modem user accesses information on the Internet, the ISPs computer (provided it handles the V.90 standard) encodes it digitally for transmission to the users PC, rather than using the modulation signalling techniques of analogue modems.  This enables downstream transmission (from ISP to users PC) at 56 Kbps.  The higher speed allows the retrieval of complex Web pages incorporating images, sound and video sequences without excessive delays.

Upstream (modem to ISP) transmissions are limited to 33.6 Kbps.  The technical reason for the lower speed is that the V.90 modem uses the conventional V.34 modulation techniques for upstream transmission.

Pulse Code Modulation (PCM)

PCM has long been used for the digitisation of audio signals, particularly the recording of music to CD, and samples an analogue waveform 8000 times per second. The diagram below illustrates this process with each vertical line representing a sampling point.

Time

PCM signal sampling

Each sample point is encoded as a separate signal.  For the journey from ISP to the users local PSTN exchange, the data travels at 64 Kbps, but is then slowed to 54 Kbps when it is converted to the analogue waveform needed for the local connection to the users PC.  The users V.90 modem can accept the PCM signals and convert them to the digital form required by the computer.

All connections via analogue modems are subject to variations in line conditions and the connect speed will frequently be below the modems maximum speed.  Connections such as ISDN and ADSL provide a much quicker and more reliable service.