Synchronous Serial Transmission

The start and stop bits used in asynchronous transmission are wasteful, as they do not contain information.  With higher speed devices or buffered (memory for the accumulation of data before it is transmitted) low-speed devices, data can be transmitted in more efficient, timed, or synchronous blocks.

The diagram below illustrates Synchronous character format.

SYN

SYN

SYN

SYN

Data block

End                                 Start

Most modems can operate synchronously or asynchronously, as required.  In synchronous transmission, a data stream may be very long, so it is vital that the bit timing between transmitter and receiver is synchronised and that individual characters are separated.

This can be achieved by using an additional transmission path to copy the transmitters timings to the receiver.  Alternatively, timing signals can be embedded within the data stream.  These timing signals, referred to as SYN characters are placed at the beginning of each data block and at intervals within the block.  If timing is lost, the receiver can re-time its bit groupings from the last received SYN character.

Like the start and stop bits used in asynchronous transmission, SYN characters constitute an overhead and have to be stripped out by the receiver.  Some computer terminals are designed for high-speed data transmission and use synchronous transmission, but many others use asynchronous transmission.

The data in a stream may consist of discrete text characters, coded in a character code such as ASCII, or it may be pure binary.  Examples of pure binary data are found in graphic images, executable machine-code programs and numbers in floating point form.  For character data streams, the allocation of a particular bit pattern to act, for example as a SYN character is not difficult.  For pure binary data, the binary patterns within the data blocks are not predictable and cannot, therefore, include embedded and unique SYN characters.  HDLC (High-Level Data Link Control) is an example of a transmission protocol which is designed to overcome this problem.

HDLC Frame Structure

Each data block occupies a frame or packet which may vary in size, typically up to 4 Kb.  An important standard for synchronous communications uses the HDLC frame structure, illustrated below.

Flag    Address   Control                               Data                                         CRC  CRC   Flag

HDLC frame structure

The frame components are explained as follows:

- The flag fields mark the limits of the frame with the binary sequence 01111110.  To ensure that this sequence always appears at the start of each frame and nowhere else, the transmitter inserts additional zeros if the reserved flag sequence happens to occur within the data area.  This is known as bit stuffing or zero insertion.  The receiver strips out these extra zeros as part of the frame unpacking process.  If a communication protocol uses a fixed frame size, bit stuffing may be used to extend frames which are shorter than the required length.

- The address field occupies one or more bytes (1 byte = 8 bits) and contains the address of the destination device, or sometimes of the sending device.  For example, when a central server wishes to communicate with an attached workstation, it will add the workstations address to the frame.  If the workstations only communicate with the central server and never directly with one another, then a transmitting workstation need only add its own address to the frame.

- The control field also occupies one or more bytes and identifies the type of frame.  Apart from containing user data, some special frames may have a Supervisory control function, perhaps to control the flow of data across the network.  The field also contains a sequence number to ensure that frames are not lost in transmission.

- The data field has no particular format and is simply the collection of bits which form the particular user data.

- The CRC (Cyclic Redundancy Check) fields contain the results of an arithmetic calculation carried out on all the bits enclosed within the flag fields.  The receiver carries out the same calculation and if it obtains the same result, it is almost certain that the contents of the frame have not been corrupted.  If a CRC error is found, the frame is discarded and the transmitter has to send it again.