##### Synchronisation of devices

If two devices are to communicate, the receiving device must run at the same speed as the sending device.  To maintain the synchronisation of the two devices, each has a clock.  The clock at the sending end tells the sending device when to transmit a bit of data on to the line (wire).  The clock at the receiving end tells the receiving device when its time to check the line for the arrival of a data bit (one or zero).  It is vital that the two clocks be kept at the same speed.

For example, if the transmitting clock sends 1200 bits of data every second and the receiving device checked the line at the rate of 1300 times per second, the result would be the detection of 100 extra bits of data which would completely disrupt the message.

#### Synchronous transmission

The easiest way to synchronise the two clocks involves the sending clock keeping the receiving clock in step.  Data has to be sent in a continuous synchronised stream, or in large blocks.  This is not always possible, or even necessary, as it requires another wire fitted between the transmitter and receiver.

An advantage of using the synchronous mode is that it’s less prone to distortion therefore usable at higher transmission speeds.

This method cannot be used for connecting computers to dial-up modems.

The diagram below shows a simplified version of synchronous transmission.

Long distance cable

Transmit clock

R   D

E   E

C   V

E    I

I     C

V   E

E

R

S     D

E    E

N    V

D     I

I      C

N     E

G

10101010

Transmit Data

Asynchronous transmission

The most common methods of connecting PCs is the asynchronous method.  A character being sent in real time via a keyboard, for example, could be sent at any moment in time.  For the system to be able to process a character, the hardware has to be told when a character is about to be received and when the transmission of the character has ceased.  This involves enclosing the bits of data with extra bits called START and STOP bits.  The START bits take the signal off its idle state, which in turn starts the clock.  Incoming bits at the receiving end can now be sampled at the clock rate.  The STOP bit returns the system back to the idle state.

#### Advantages

- Requires fewer connection wires.

- Works well with irregular data streams – e.g. keyboard entry transmissions.

Disadvantages

- START bits can be missed, sometimes caused by interference on the wire.  This can lead to data bits being misread as START bits thus producing errors.

###### This method is slow because some of the bits transmitted are for control purposes only.  For example, a system with 2 START bits and 1 STOP bit would require 11 bits to transfer a single byte of user data.