BTEC NAT – UNIT 5 – NETWORKING

HARDWARE & CABLING

## What is Networking Hardware?

Networking hardware includes all computers, peripherals, interface cards and other equipment needed to perform data-processing and communications within the network.

## File Servers

A file server stands at the heart of most networks. It is a very fast computer with a large amount of RAM and storage space, along with a fast network interface card. The network operating system software resides on this computer, along with any software applications and data files that need to be shared.

The file server controls the communication of information between the nodes on a network. For example, it may be asked to send a word processor program to one workstation, receive a database file from another workstation, and store an e-mail message during the same time period. This requires a computer that can store a lot of information and share it very quickly. File servers should have at least the following characteristics:

- Fast microprocessor (Pentium, PowerPC)

- A fast hard drive with at least nine gigabytes of storage

- A RAID (Redundant Array of Inexpensive Disks) to preserve data after a disk casualty

- A tape back-up unit (i.e. DAT, JAZ, Zip, or CD-RW drive)

- Numerous expansion slots

- Fast network interface card

- At least of 32 MB of RAM

## Workstations

All of the computers connected to the file server on a network are called workstations. A typical workstation is a computer that is configured with a network interface card, networking software, and the appropriate cables. Workstations do not necessarily need floppy disk drives or hard drives because files can be saved on the file server. Almost any computer can serve as a network workstation.

## Network Interface Cards

The network interface card (NIC) provides the physical connection between the network and the computer workstation. Most NICs are internal, with the card fitting into an expansion slot inside the computer. Some computers, such as Mac Classics, use external boxes which are attached to a serial port or a SCSI port. Laptop computers can now be purchased with a network interface card built-in or with network cards that slip into a PCMCIA slot.

Network interface cards are a major factor in determining the speed and performance of a network. It is a good idea to use the fastest network card available for the type of workstation you are using.

The three most common network interface connections are Ethernet cards, LocalTalk connectors, and Token Ring cards.

### Ethernet Cards

Ethernet cards are usually purchased separately from a computer, although many computers (such as the Macintosh) now include an option for a pre-installed Ethernet card. Ethernet cards contain connections for either coaxial or twisted pair cables (or both) (See fig. 1). If it is designed for coaxial cable, the connection will be BNC. If it is designed for twisted pair, it will have a RJ-45 connection. Some Ethernet cards also contain an AUI connector. This can be used to attach coaxial, twisted pair, or fibre optics cable to an Ethernet card. When this method is used there is always an external transceiver attached to the workstation. (See the Cabling section for more information on connectors.)

Fig. 1. Ethernet card.
From top to bottom:
RJ-45, AUI, and BNC connectors

### LocalTalk Connectors

LocalTalk is Apple's built-in solution for networking Macintosh computers. It utilizes a special adapter box and a cable that plugs into the printer port of a Macintosh. A major disadvantage of LocalTalk is that it is slow in comparison to Ethernet. Most Ethernet connections operate at 10 Mbps (Megabits per second). In contrast, LocalTalk operates at only 230 Kbps (or .23 Mbps).

### Token Ring Cards

Token Ring network cards look similar to Ethernet cards. One visible difference is the type of connector on the back end of the card. Token Ring cards generally have a nine pin DIN type connector to attach the card to the network cable.

## Switch

A concentrator is a device that provides a central connection point for cables from workstations, servers, and peripherals. In a star topology, twisted-pair wire is run from each workstation to a central switch/hub. Most switches are active, that is they electrically amplify the signal as it moves from one device to another. Switches no longer broadcast network packets as hubs did in the past, they memorize addressing of computers and send the information to the correct location directly. Switches are:

- Usually configured with 8, 12, or 24 RJ-45 ports

- Often used in a star or star-wired ring topology

- Sold with specialized software for port management

- Also called hubs

- Usually installed in a standardized metal rack that also may store net modems, bridges, or routers

## Repeaters

Since a signal loses strength as it passes along a cable, it is often necessary to boost the signal with a device called a repeater. The repeater electrically amplifies the signal it receives and rebroadcasts it. Repeaters can be separate devices or they can be incorporated into a concentrator. They are used when the total length of your network cable exceeds the standards set for the type of cable being used.

A good example of the use of repeaters would be in a local area network using a star topology with unshielded twisted-pair cabling. The length limit for unshielded twisted-pair cable is 100 metres. The most common configuration is for each workstation to be connected by twisted-pair cable to a multi-port active concentrator. The concentrator amplifies all the signals that pass through it allowing for the total length of cable on the network to exceed the 100 metre limit.

## Bridges

A bridge is a device that allows you to segment a large network into two smaller, more efficient networks. If you are adding to an older wiring scheme and want the new network to be up-to-date, a bridge can connect the two. A bridge monitors the information traffic on both sides of the network so that it can pass packets of information to the correct location.

Most bridges can "listen" to the network and automatically figure out the address of each computer on both sides of the bridge. The bridge can inspect each message and, if necessary, broadcast it on the other side of the network. The bridge manages the traffic to maintain optimum performance on both sides of the network. You might say that the bridge is like a traffic cop at a busy intersection during rush hour. It keeps information flowing on both sides of the network, but it does not allow unnecessary traffic through. Bridges can be used to connect different types of cabling, or physical topologies. They must, however, be used between networks with the same protocol.

## Routers

A router translates information from one network to another; it is similar to a superintelligent bridge. Routers select the best path to route a message, based on the destination address and origin. The router can direct traffic to prevent head-on collisions, and is smart enough to know when to direct traffic along back roads and shortcuts. While bridges know the addresses of all computers on each side of the network, routers know the addresses of computers, bridges, and other routers on the network. Routers can even "listen" to the entire network to determine which sections are busiest -- they can then redirect data around those sections until they clear up.

If you have a college LAN that you want to connect to the Internet, you will need to purchase a router. In this case, the router serves as the translator between the information on your LAN and the Internet. It also determines the best route to send the data over the Internet.

Routers can:

Direct signal traffic efficiently

Route messages between any two protocols

Route messages between linear bus, star, and star-wired ring topologies

Route messages across fibre optic, coaxial, and twisted-pair cabling

## What is Network Cabling?

Cable is the medium through which information usually moves from one network device to another. There are several types of cable which are commonly used with LANs. In some cases, a network will utilize only one type of cable, other networks will use a variety of cable types. The type of cable chosen for a network is related to the network's topology, protocol, and size. Understanding the characteristics of different types of cable and how they relate to other aspects of a network is necessary for the development of a successful network.

## Unshielded Twisted Pair (UTP) Cable

Twisted pair cabling comes in two varieties: shielded and unshielded. Unshielded twisted pair (UTP) is the most popular and is generally the best option for school networks (See fig. 1).

Fig.1. Unshielded twisted pair

The quality of UTP may vary from telephone-grade wire to extremely high-speed cable. The cable has four pairs of wires inside the jacket. Each pair is twisted with a different number of twists per inch to help eliminate interference from adjacent pairs and other electrical devices. The tighter the twisting, the higher the supported transmission rate and the greater the cost per foot. The EIA/TIA (Electronic Industry Association/Telecommunication Industry Association) has established standards of UTP and rated five categories of wire.

### Categories of Unshielded Twisted Pair

Type; Use. Category 1; Voice Only (Telephone Wire). Category 2; Data to 4 Mbps (LocalTalk). Category 3; Data to 10 Mbps (Ethernet). Category 4; Data to 20 Mbps (16 Mbps Token Ring). Category 5; Data to 100 Mbps (Fast Ethernet).

### Unshielded Twisted Pair Connector

The standard connector for unshielded twisted pair cabling is an RJ-45 connector. This is a plastic connector that looks like a large telephone-style connector (See fig. 2). A slot allows the RJ-45 to be inserted only one way. RJ stands for Registered Jack, implying that the connector follows a standard borrowed from the telephone industry. This standard designates which wire goes with each pin inside the connector.

Fig. 2. RJ-45 connector

## Shielded Twisted Pair (STP) Cable

A disadvantage of UTP is that it may be susceptible to radio and electrical frequency interference. Shielded twisted pair (STP) is suitable for environments with electrical interference; however, the extra shielding can make the cables quite bulky. Shielded twisted pair is often used on networks using Token Ring topology.

## Coaxial Cable

Coaxial cabling has a single copper conductor at its centre. A plastic layer provides insulation between the centre conductor and a braided metal shield (See fig. 3). The metal shield helps to block any outside interference from fluorescent lights, motors, and other computers.

Fig. 3. Coaxial cable

Although coaxial cabling is difficult to install, it is highly resistant to signal interference. In addition, it can support greater cable lengths between network devices than twisted pair cable. The two types of coaxial cabling are thick coaxial and thin coaxial.

Thin coaxial cable is also referred to as thinnet. 10Base2 refers to the specifications for thin coaxial cable carrying Ethernet signals. The 2 refers to the approximate maximum segment length being 200 metres. In actual fact the maximum segment length is 185 metres. Thin coaxial cable is popular in school networks, especially linear bus networks.

Thick coaxial cable is also referred to as thicknet. 10Base5 refers to the specifications for thick coaxial cable carrying Ethernet signals. The 5 refers to the maximum segment length being 500 metres. Thick coaxial cable has an extra protective plastic cover that helps keep moisture away from the centre conductor. This makes thick coaxial a great choice when running longer lengths in a linear bus network. One disadvantage of thick coaxial is that it does not bend easily and is difficult to install.

### Coaxial Cable Connectors

The most common type of connector used with coaxial cables is the Bayone-Neill-Concelman (BNC) connector (See fig. 4). Different types of adapters are available for BNC connectors, including a T-connector, barrel connector, and terminator. Connectors on the cable are the weakest points in any network. To help avoid problems with your network, always use the BNC connectors that crimp, rather than screw, onto the cable.

Fig. 4. BNC connector

## Fibre Optic Cable

Fibre optic cabling consists of a centre glass core surrounded by several layers of protective materials (See fig. 5). It transmits light rather than electronic signals eliminating the problem of electrical interference. This makes it ideal for certain environments that contain a large amount of electrical interference. It has also made it the standard for connecting networks between buildings, due to its immunity to the effects of moisture and lighting.

Fibre optic cable has the ability to transmit signals over much longer distances than coaxial and twisted pair. It also has the capability to carry information at vastly greater speeds. This capacity broadens communication possibilities to include services such as video conferencing and interactive services. The cost of fibre optic cabling is comparable to copper cabling; however, it is more difficult to install and modify. 10BaseF refers to the specifications for fibre optic cable carrying Ethernet signals.

Fig.5. Fibre optic cable

### Fibre Optic Connector

The most common connector used with fibre optic cable is an ST connector. It is barrel shaped, similar to a BNC connector. A newer connector, the SC, is becoming more popular. It has a squared face and is easier to connect in a confined space.

### Ethernet Cable Summary

Specification; Cable Type; Maximum length. 10BaseT; Unshielded Twisted Pair; 100 metres. 10Base2; Thin Coaxial; 185 metres. 10Base5; Thick Coaxial; 500 metres. 10BaseF; Fibre Optic; 2000 metres. 100BaseT; Unshielded Twisted Pair; 100 metres. 100BaseTX; Unshielded Twisted Pair; 220 metres.

## Wireless LANs

Not all networks are connected with cabling; some networks are wireless. Wireless LANs use high frequency radio signals, infrared light beams, or lasers to communicate between the workstations and the file server or hubs. Each workstation and file server on a wireless network has some sort of transceiver/antenna to send and receive the data. Information is relayed between transceivers as if they were physically connected. For longer distance, wireless communications can also take place through cellular telephone technology, microwave transmission, or by satellite.

Wireless networks are great for allowing laptop computers or remote computers to connect to the LAN. Wireless networks are also beneficial in older buildings where it may be difficult or impossible to install cables.

The two most common types of infrared communications used in colleges are line-of-sight and scattered broadcast. Line-of-sight communication means that there must be an unblocked direct line between the workstation and the transceiver. If a person walks within the line-of-sight while there is a transmission, the information would need to be sent again. This kind of obstruction can slow down the wireless network.

Scattered infrared communication is a broadcast of infrared transmissions sent out in multiple directions that bounces off walls and ceilings until it eventually hits the receiver. Networking communications with laser are virtually the same as line-of-sight infrared networks.

Wireless LANs have several disadvantages. They are very expensive, provide poor security, and are susceptible to interference from lights and electronic devices. They are also slower than LANs using cabling.