PRINCIPLES OF NETWORKING

SOFTWARE

## What is a Network Operating System?

Unlike operating systems, such as DOS and Windows, that are designed for single users to control one computer, network operating systems (NOS) coordinate the activities of multiple computers across a network. The network operating system acts as a director to keep the network running smoothly.

The two major types of network operating systems are:

## Peer-to-Peer

Peer-to-peer network operating systems allow users to share resources and files located on their computers and to access shared resources found on other computers. However, they do not have a file server or a centralized management source. In a peer-to-peer network, all computers are considered equal; they all have the same abilities to use the resources available on the network. Peer-to-peer networks are designed primarily for small to medium local area networks. AppleShare and Windows for Workgroups are examples of programs that can function as peer-to-peer network operating systems.

### Advantages of a peer-to-peer network:

- Less initial expense - No need for a dedicated server.

- Setup - An operating system (such as Windows XP) already in place may only need to be reconfigured for peer-to-peer operations.

### Disadvantages of a peer-to-peer network:

- Decentralized - No central repository for files and applications.

- Security - Does not provide the security available on a client/server network.

## Client/Server

Client/server network operating systems allow the network to centralize functions and applications in one or more dedicated file servers (See fig. 2). The file servers become the heart of the system, providing access to resources and providing security. Individual workstations (clients) have access to the resources available on the file servers. The network operating system provides the mechanism to integrate all the components of the network and allow multiple users to simultaneously share the same resources irrespective of physical location. Novell Netware and Windows 2000 Server are examples of client/server network operating systems.

Fig. 2. Client/server network

### Advantages of a client/server network:

- Centralized - Resources and data security are controlled through the server.

- Scalability - Any or all elements can be replaced individually as needs increase.

- Flexibility - New technology can be easily integrated into system.

- Interoperability - All components (client/network/server) work together.

- Accessibility - Server can be accessed remotely and across multiple platforms.

### Disadvantages of a client/server network:

- Expense - Requires initial investment in dedicated server.

- Maintenance - Large networks will require a staff to ensure efficient operation.

- Dependence - When server goes down, operations will cease across the network.

## Examples of network operating systems

The following list includes some of the more popular peer-to-peer and client/server network operating systems.

- Apple Share

- LANtastic

- Microsoft Windows 2000 Server

- Novell Netware

NETWORKING GLOSSARY

10Base2 - Ethernet specification for thin coaxial cable, transmits signals at 10 Mbps (megabits per second) with a distance limit of 185 metres per segment.

10Base5 - Ethernet specification for thick coaxial cable, transmits signals at 10 Mbps (megabits per second) with a distance limit of 500 metres per segment.

10BaseF - Ethernet specification for fibre optic cable, transmits signals at 10 Mbps (megabits per second) with a distance limit of 2000 metres per segment.

10BaseT - Ethernet specification for unshielded twisted pair cable (category 3, 4, or 5), transmits signals at 10 Mbps (megabits per second) with a distance limit of 100 metres per segment.

100BaseT - Ethernet specification for unshielded twisted pair cabling that is used to transmit data at 100 Mbps (megabits per second) with a distance limit of 100 metres per segment.

100BaseTX -Ethernet specification for unshielded twisted pair cabling that is used to transmit data at 1 Gbps (gigabits per second) with a distance limitation of 220 metres per segment.

Asynchronous Transfer Mode (ATM) - A network protocol that transmits data at a speed of 155 Mbps and higher. It is most often used to interconnect two or more local area networks.

AppleTalk - Apple Computer's network protocol originally designed to run over LocalTalk networks, but can also run on Ethernet and Token Ring.

AUI Connector (Attachment Unit Interface) - A 15 pin connector found on Ethernet cards that can be used for attaching coaxial, fibre optic, or twisted pair cable.

Backbone - A cable to which multiple nodes or workstations are attached.

Bit - Binary digit in the binary numbering system. Its value can be 0 or 1. In an 8-bit character scheme, it takes 8 bits to make a byte (character) of data.

BNC Connector (Bayone-Neill-Concelman) - Standard connector used to connect 10Base2 coaxial cable.

Bridge - Devices that connect and pass packets between two network segments that use the same communications protocol.

Cable - Transmission medium of copper wire or optical fibre wrapped in a protective cover.

Client/Server - A networking system in which one or more file servers (Server) provide services; such as network management, application and centralized data storage for workstations (Clients).

CSMA/CA - Carrier Sense Multiple Access Collision Avoidance is a network access method in which each device signals its intent to transmit before it actually does so. This prevents other devices from sending information, thus preventing collisions from occurring between signals from two or more devices. This is the access method used by LocalTalk.

CSMA/CD - Carrier Sense Multiple Access Collision Detection is a network access method in which devices that are ready to transmit data first check the channel for a carrier. If no carrier is sensed, a device can transmit. If two devices transmit at once, a collision occurs and each computer backs off and waits a random amount of time before attempting to retransmit. This is the access method used by Ethernet.

Coaxial Cable - Cable consisting of a single copper conductor in the centre surrounded by a plastic layer for insulation and a braided metal outer shield.

Concentrator - A device that provides a central connection point for cables from workstations, servers, and peripherals. Most concentrators contain the ability to amplify the electrical signal they receive.

DIN - A plug and socket connector consisting of a circular pattern of pins in a metal sleeve. This type of connector is commonly seen on keyboards.

Dumb Terminal - Refers to devices that are designed to communicate exclusively with a host (main frame) computer. It receives all screen layouts from the host computer and sends all keyboard entry to the host. It cannot function without the host computer.

E-mail - An electronic mail message sent from a host computer to a remote computer.

End User - Refers to the human executing applications on the workstation.

Ethernet - A network protocol invented by Xerox Corporation and developed jointly by Xerox, Intel and Digital Equipment Corporation. Ethernet networks use CSMA/CD and run over a variety of cable types at 10 Mbps (megabits per second).

Expansion Slot - Area in a computer that accepts additional input/output boards to increase the capability of the computer.

Fast Ethernet - A new Ethernet standard that supports 100 Mbps using category 5 twisted pair or fibre optic cable.

Fibre Distributed Data Interface (FDDI) - A network protocol that is used primarily to interconnect two or more local area networks, often over large distances.

Fibre Optic Cable - A cable, consisting of a centre glass core surrounded by layers of plastic, that transmits data using light rather than electricity. It has the ability to carry more information over much longer distances.

File Server - A computer connected to the network that contains primary files/applications and shares them as requested with the other computers on the network. If the file server is dedicated for that purpose only, it is connected to a client/server network. An example of a client/server network is Novell Netware. All the computers connected to a peer-to-peer network are capable of being the file server. Two examples of peer-to-peer networks are LANtastic and Windows for Workgroups.

Gigabit Ethernet - An Ethernet protocol that raises the transmission rates to 1 Gbps (gigabits per second). It is primarily used for a high speed backbone of a network.

Gigabyte (GB) - One billion bytes of information. One thousand megabytes.

Hub - A hardware device that contains multiple independent but connected modules of network and internetwork equipment. Hubs can be active (where they repeat signals sent through them) or passive (where they do not repeat but merely split signals sent through them).

Infrared - Electromagnetic waves whose frequency range is above that of microwaves, but below that of the visible spectrum.

Intranet - Network internal to an organization that uses Internet protocols.

Internet - A global network of networks used to exchange information using the TCP/IP protocol. It allows for electronic mail and the accessing ad retrieval of information from remote sources.

LAN (Local Area Network) - A network connecting computers in a relatively small area such as a building.

Linear Bus - A network topology in which each node attaches directly to a common cable.

LocalTalk - Apple Corporation proprietary protocol that uses CSMA/CA media access scheme and supports transmissions at speeds of 230 Kbps (Kilobits per second).

MAN (Metropolitan Area Network) - A network connecting computers over a large geographical area, such as a city or school district.

MAU (Multistation Access Unit) - A Token Ring wiring hub.

Modem (Modulator/Demodulator) - Devices that convert digital and analogue signals. Modems allow computer data (digital) to be transmitted over voice-grade telephone lines (analogue).

Multiplexer - A device that allows multiple logical signals to be transmitted simultaneously across a single physical channel.

Network Modem - A modem connected to a Local Area Network (LAN) that is accessible from any workstation on the network.

Network Interface Card (NIC) - A board that provides network communication capabilities to and from a computer.

Network Operating System (NOS) - Operating system designed to pass information and communicate between more than one computer. Examples include AppleShare, Novell NetWare, and Windows NT Server.

Node - End point of a network connection. Nodes include any device attached to a network such as file servers, printers, or workstations.

Node Devices - Any computer or peripheral that is connected to the network.

PCMCIA - An expansion slot found in many laptop computers.

Peer-to-Peer Network - A network in which resources and files are shared without a centralized management source.

Physical Topology - The physical layout of the network; how the cables are arranged; and how the computers are connected.

Point-to-Point - A direct link between two objects in a network.

Ports - A connection point for a cable.

Protocol -A formal description of a set of rules and conventions that govern how devices on a network exchange information.

RAID (Redundant Array of Inexpensive Disks) - A configuration of multiple disks designed to preserve data after a disk casualty.

RAM (Random Access Memory) - The working memory of a computer where data and programs are temporarily stored. RAM only holds information when the computer is on.

Repeater - A device used in a network to strengthen a signal as it is passed along the network cable.

RJ-45 - Standard connectors used for unshielded twisted-pair cable.

Router -A device that routes information between interconnected networks. It can select the best path to route a message, as well as translate information from one network to another. It is similar to a superintelligent bridge.

SCSI (Small Computer Serial Interface) - An interface controller that allows several peripherals to be connected to the same port on a computer.

Segment - Refers to a section of cable on a network. In Ethernet networks, two types of segments are defined. A populated or trunk segment is a network cable that has one or more nodes attached to it. A link segment is a cable that connects a computer to an interconnecting device, such as a repeater or concentrator, or connects a interconnecting device to another interconnecting device.

Sneaker-Net - Refers to a manual method of sharing files in which a file is copied from a computer to a floppy disk, transported to a second computer by a person physically walking (apparently wearing sneakers) to the second computer, and manually transferring the file from floppy disk to the second computer.

Speed of Data Transfer - The rate at which information travels through a network, usually measured in megabits per second.

Star Topology - LAN topology in which each node on a network is connected directly to a central network hub or concentrator.

Star-Wired Ring - Network topology that connects network devices (such as computers and printers) in a complete circle.

Tape Back-Up - Copying all the data and programs of a computer system on magnetic tape. On tape, data is stored sequentially. When retrieving data, the tape is searched from the beginning of tape until the data is found.

Terminator - A device that provides electrical resistance at the end of a transmission line. Its function is to absorb signals on the line, thereby keeping them from bouncing back and being received again by the network.

Thicknet - A thick coaxial cable that is used with a 10Base5 Ethernet LAN.

Thinnet - A thin coaxial cable that is used with a 10Base2 Ethernet LAN.

Token - A special packet that contains data and acts as a messenger or carrier between each computer and device on a ring topology. Each computer must wait for the messenger to stop at its node before it can send data over the network.

Token Ring - A network protocol developed by IBM in which computers access the network through token-passing. Usually uses a star-wired ring topology.

Topology - There are two types of topology: physical and logical. The physical topology of a network refers to the configuration of cables, computers, and other peripherals. Logical topology is the method used to pass the information between workstations. Issues involving logical topologies are discussed on the Protocol chapter

Transceiver (Transmitter/Receiver) - A Device that receives and sends signals over a medium. In networks, it is generally used to allow for the connection between two different types of cable connectors, such as AUI and RJ-45.

Tree Topology - LAN topology similar to linear bus topology, except that tree networks can contain branches with multiple nodes.

Twisted Pair - Network cabling that consists of four pairs of wires that are manufactured with the wires twisted to certain specifications. Available in shielded and unshielded versions.

USB (Universal Serial Bus) Port - A hardware interface for low-speed peripherals such as the keyboard, mouse, joystick, scanner, printer, and telephony devices.

WAN (Wide Area Network) - A network connecting computers within very large areas, such as states, countries, and the world.

Workgroup - A collection of workstations and servers on a LAN that are designated to communicate and exchange data with one another.

Workstation - A computer connected to a network at which users interact with software stored on the network.